APPLICATION PROGRAMMING INTERFACE

of “Dunay” monitoring system

1

2

This document is a guide for the programmer performing the interaction with the FPGA (field- programmable gate array).

This document provides information describing control commands, responses to them, as well as the structure of input and output data of “Dunay” device.

3

Contents

1. Conditions and terms of use ............................................................. 4

2.1. Firmware ....................................................................................... 5

3. ОБРАЩЕНИЕ ................................................................................. 6

3.1. Общие сведения ........................................................................... 6

3.2. Структура команд ........................................................................ 6

4. ВХОДНЫЕ И ВЫХОДНЫЕ ДАННЫЕ ........................................ 9

4.1. Общие сведения ........................................................................... 9

4.2. Структура данных ........................................................................ 9

5. СООБЩЕНИЯ ................................................................................ 11

5.1. Общие сведения ........................................................................... 11

5.2. Структура откликов на команды ................................................ 11

Приложение 1 Структура регистров ................................................. 14

Перечень принятых сокращений ................................. Ошибка! Закладка не определена.

4

1. Conditions and terms of use

1.1. This protocol allows:

- to control the device (to form and change system parameters);

- to receive rough phase data from the device.

1.2. The interaction with Dunay system takes place via a personal computer (PC) via an Ethernet

connection.

To work with the device:

- the information interaction between the PC and Dunay system should be connected within the lo-

cal area network based on the TCP / IP protocol with a bandwidth of at least 1 Gbit / s (the PC must have

an Ethernet network interface with a bandwidth of at least 1 Gbit / s);

5

2. Device features

2.1. Firmware

2.1.1. Flash memory stores two firmwares: "factory" and "application".

The "factory" firmware is used:

- at power-up;

- if the error of the firmware "application" occurs;

- on command from the firmware "application".

The firmware "application" is loaded:

- from the "factory" firmware by command.

2.1.2. «Factory» firmware.

- contains the interface of the exchange with the PC;

- contains the exchange interface with the microcontroller;

- allows to manage the Flash-memory configuration;

- on the external command can download the firmware "application" with the integrity check;

- contains the number of its version.

2.1.3. Firmware "application":

- contains the base functionality of signal processing and an extended interface with PC;

- does not allow to manage the configuration Flash-memory.

- on the external command can download the firmware "factory".

6

3. Connection

3.1. General information

3.1.1. After connecting the Sava module to the PC, it is recommended to make sure that it works as

required:

- Make sure that the connection to the Sava module is established using the "ping" command.

3.1.2. The interaction takes place over the network protocol UDP, which is described below.

The fact of acceptance of the management command (Table 2) is confirmed by the response to it

(Table 9).

3.2. Command structure

3.2.1. Interaction with the product is done using commands. The general structure of the commands

is shown in Table 2.

Table 1 – Command structure

| Description | Size, Byte | Comments |
| --- | --- | --- |
| Command code 0x0000 XXXX | 4 |  |
| Data | N | If available |

3.2.2. The complete list of commands is presented in Table 2.

Note: Before sending any control command from the PC to the "Dunay", you must send a stop

command (Command code: 0x0000 0003).

Table 2 – Complete list of commands

| Description | Command code | Data Size, Byte | Data Description |
| --- | --- | --- | --- |
| Control commands |  |  |  |
| Stopping data transfer over UDP | 0x0000 0003 | – |  |
| Command 32-bit registers | 0x0000 0006 | – |  |
| Installing 32-bit registers (Appendix 1) | 0x0000 0007 | 16×4=64 | Register values (appendix 1) |
| Setting the IP address of the data recipient | 0x0000 0008 | 4 | IP-address |
| Commanding the IP address of the data receiver | 0x0000 0009 | – |  |

7

| Description | Command code | Data Size, Byte | Data Description |
| --- | --- | --- | --- |
| Mode "Reflectogram" | 0x0000 0011 | – |  |
| Filters mode | 0x0000 0015 | – |  |
| Firmware version command | 0x0000 0017 | – |  |
| Transmitting the coordinates (the distance from the beginning of the fiber optic cable) of the points for sound removal for 32-bit registers (Appendix 1) | 0x0000 0018 | 4 4 | Coordinate of the first point Coordinate of the second point |
| Command three thresholds for detection | 0x0000 0306 | – |  |
| Setting three thresholds for detection | 0x0000 0307 | 4 4 4 | Threshold of the 60–150 Hz Channel Threshold of the 20–40 Hz Channel Threshold of the 4–10 Hz Channel |
| Mode "Transfer Phase" | 0x0000 0308 | – |  |
| Mode "Transfer of the phase together and the filtered phase envelope" | 0x0000 0309 | – |  |
| Remote boot commands FPGA |  |  |  |
| Go to the firmware "application" 1) | 0x0000 0100 | – |  |
| Go to the "factory" firmware 1) | 0x0000 0101 | – |  |
| Configuration Flash Management Commands |  |  |  |
| Commanding memory size | 0x0000 0a00 | – |  |
| Erase | 0x0000 0800 | – |  |
| Recording 2) | 0x0000 0100 | 4 1460 | address data |
| Reading 2) | 0x0000 0c00 | 4 | address [uint32] |

3.2.3. The data transmission and reception ports are shown in Table 3.

Table 3 – Transmission and reception ports

| № | Holder | Purpose |
| --- | --- | --- |
| Control commands |  |  |
| 8201 | Device | Receiving commands (command code + data) |

8

| 8202 | PC | Receiving data in the "Reflectogram" mode |
| --- | --- | --- |
| 8207 | PC | Receiving data in the "Filters" mode (filter 60–150 Hz) |
| 8208 | PC | Receiving data in the "Filters" mode (filter 20–40 Hz) |
| 8209 | PC | Receiving data in the "Filters" mode (filter 4–10 Hz) |
| 8227 | PC | Receiving data in the "Phase" mode |
| 8228 | PC | Receiving data in the "Phase and Filters" mode |
| 8211 | PC | Reception of responses to commands (response code + data) |
| Configuration Flash Management Commands |  |  |
| 9229 | Device | Receiving commands (command code + data) |
| 9230 | PC | Reception of responses to commands (response code + data) |
| 9231 | PC | Receiving data |

9

4. Input and output data

4.1. General information

4.1.1. The product works in the following modes:

- "Reflectogram" mode - transfer of raw data from the ADC to the PC;

- "Filters" mode - transfer of filtered data from the ADC to the PC;

- "Phase" mode - transfer of processed data from the ADC to a PC in a certain way;

- "Phase and filters" mode;

- "Waiting" mode - stop transfer of data from the ADC to the PC to change the mode (command 0x0000 0003).

4.2. Data structure

4.2.1. The data from the product is transferred to the PC in packets of fixed length (1478 bytes for

the UDP packet).

4.2.2. The structure of the parcels when transmitting data in the "Reflectogram" mode is given in

Table 4.

Table 4 – Structure of the parcels when transmitting data in the "Reflectogram"

| Description | Size, byte | Comments |
| --- | --- | --- |
| Number of the parcel (reflectogram) | 2 |  |
| Package number (parts of the trace) | 2 |  |
| ADC data | N | ADC data is a consecutive array of non-negative numbers of 2 bytes (16 bits) N - not more than 1464. |
| Zeros | 1474−N | The device itself adds zeros after the data (instead of data if there is none) to achieve the desired length when forming the UDP packet. |

4.2.3. The structure of the data transmissions in the "Filters" mode is shown in Table 5.

Note - Data is transmitted in fixed length packets for each of the three filtering channels in parallel

to a separate port. Depending on the number of samples, the last premise may be different from the previous ones: the last parcels are not filled with target values, and the latter cannot be filled completely. It is necessary to track the total number of samples in order to weed out zeros from the last premise. Table 5 – Structure of the data is transmitted in the "Filters" mode

10

| Description | Size, byte | Comments |
| --- | --- | --- |
| Processing cycle number | 2 |  |
| Package number | 2 |  |
| Envelope values for 30 points of a line | 1440 | For each point, 24 16-bit numbers are transmitted |
| Zeros | 34 | The device itself adds zeros after the data (instead of data if there is none) to achieve the desired length when forming the UDP packet. |

4.2.4. The structure of the parcels when transmitting data in the "Phase" or "Phase and Filters"

mode is given in Table 6.

Table 6 – The structure of the transmission when transmitting data in the "Phase" or "Phase and Filters" mode

| Description | Size, byte | Comments |
| --- | --- | --- |
| Line point number | 4 |  |
| Block number | 4 |  |
| Radius-vector components | 4 | One first time point is transmitted for the value of the radius vector for 500 phase values. |
| Data (differential phase), for the current point - 500 samples of 2 bytes each | 1000 | For a variant with averaging 8 the component for 8 points of the line is averaged. |
| The data (24 samples 2 bytes each) of the envelope after the bandpass filter 60–150 Hz | 48 | When sending a phase in a packet, the phase data from one point is decimated 4 times |
| The data (24 samples 2 bytes each) of the envelope after the bandpass filter 20–40 Hz | 48 | When sending a phase in a packet, the phase data from one point is decimated 4 times |

11

| Description | Size, byte | Comments |
| --- | --- | --- |
| The data (24 samples 2 bytes each) of the envelope after the bandpass filter 4–10 Hz | 48 | When sending a phase in a packet, the phase data from one point is decimated 4 times |

5. Messages

5.1. General information

5.1.1. The fact of acceptance of the command is confirmed by a response to it.

5.1.2. The device, responding to the request, sends data to the same IP address from which this re-

quest came.

5.2. Structure of responses to commands

5.2.1. General structure of responses to commands is in table 8.

Table 8 – Structure of responses to commands

| Description | Size, byte | Comments |
| --- | --- | --- |
| Response to command 0x0000 XXXX | 4 | The response to the supported command is 0xF001 XXXX The response to the unsupported command is 0xF002 XXXX Response to the register request command 0x0001 0006 |
| Data | N | If available |
| Zeros | 14−N | The system itself adds zeros after the data (instead of data if there is none) to achieve the desired length when forming the UDP packet. In this case, in the response 0xF001 0006 the data length is 1474 bytes |

5.2.2. The complete list of responses to commands is presented in Table 9.

Table 9 – Complete list of responses to commands

| Command description | Response code | Data Size, byte | Data Description |
| --- | --- | --- | --- |
| Control commands |  |  |  |

12

| Command description | Response code | Data Size, byte | Data Description |
| --- | --- | --- | --- |
| Stopping data transfer over UDP | 0xF001 0003 | – |  |
| Request 32-bit registers | 0x0001 0006 | 64 | Values of registers (appendix 1) |
| Setting 32-bit registers | 0xF001 0007 | – |  |
| Setting the IP address of the data recipient | 0xF001 0008 | – |  |
| Requesting the IP address of the data receiver | 0xF001 0009 | 4 | IP-address |
| Mode "Reflectogram" | 0xF001 0011 | – |  |
| "Sound transmission" mode | 0xF001 0012 | – |  |
| Filters mode | 0xF001 0015 | – |  |
| Firmware version request | 0xF001 0017 | 4 4 | Device № Firmware version |
| Transfer of coordinates of points for sound removal for 32-bit registers | 0xF001 0018 | – |  |
| Request three thresholds for detection | 0xF001 0306 | 4 4 4 | Threshold of the 60–150 Hz Channel Threshold of the 20–40 Hz Channel Threshold of the 4–10 Hz Channel |
| Setting three thresholds for detection | 0xF001 0307 | – |  |
| Mode "Transfer Phase" | 0xF001 0308 | – |  |
| Mode "Transfer of the phase together and the filtered phase envelope" | 0xF001 0309 | – |  |
| Remote boot commands FPGA |  |  |  |
| Go to the firmware "application" 1) | 0xF001 0100 | – |  |
| Go to the "factory" firmware 1) | 0xF001 0101 | – |  |
| Configuration Flash Management Commands |  |  |  |
| Answer size of memory | 0x0000 0a00 | 4 | Memory size [uint32] |
| Confirmation of erasure | 08h0x0000 0800 | – |  |
| Confirmation of a record | 01h0xF001 0100 | – |  |
| Read Confirmation | 0x0000 0c00 | 4 1460 | address data |

13

| Command description | Response code | Data Size, byte | Data Description |
| --- | --- | --- | --- |
| Sending ready | 02h0x0000 0200 | – |  |

14

Appendix 1

Registers structure

Table 10 – Registers structure

| Shift, byte | Name | Size, byte | Description |
| --- | --- | --- | --- |
| 0 | The period of the scanning pulses | 4 | The time interval between the sending of laser pulses in a line in tens of nanoseconds from 8192 (12 kHz) to 1048576 (~100 Hz), but not less than the time for the signal to pass the entire line |
| 4 | Modulator 1 pulse width | 4 | The pulse duration in dozens of nanoseconds from 1 to 65535 |
| 8 | The pulse of modulator 2 begins | 4 | Not used |
| 12 | End of Modulator 2 pulse | 4 | Not used |
| 16 | Start reading the trace | 4 | The coordinate from which the transmission of the waveforms from the device begins and the display in meters. Counted from the beginning of the line |
| 20 | End of reading the trace | 4 | The coordinate, which ends the transmission of the reflectograms from the device and the display in meters. Counted from the beginning of the line |
| 24 | Number of traces to be skipped | 4 | from 1 to 255 |
| 28 | The time threshold for the detection function (the number of thresholds to be exceeded) | 4 | Not used |
| 32 | Number of points for smoothing by time | 4 | Not used |
| 36 | The first point of sound removal | 4 | in the range of reading the reflectogramm |
| 40 | The second point of sound removal | 4 | in the range of reading the reflectogramm |
| 44 | Filter Control Register | 4 | Table 11 |

15

| Shift, byte | Name | Size, byte | Description |
| --- | --- | --- | --- |
| 48 | Time threshold of the detection function (number of passes in the threshold exceeding section) | 4 | Not used |
| 52 | Line Start | 4 | The distance from the product to the point from which the distance starts (for example, the exit of the fiber cable from the building). This parameter also contains a constant delay of the probing signal in the instrument. |
| 56 | The decimation factor along the line | 4 | from 1 to 255 |
| 60 | Flag thinning anti-aliasing (0/1) | 4 | 0,1 |
| – | Reserved | 1426 | – |

Table 11 – Description of the control register "Filter control register"

| Bit number | Description |
| --- | --- |
| 10 | Test sequence 1 (test sequences are different for different firmware) |
| 11 | Test sequence 2 (test sequences differ for different firmware) |
| 12 | must always be "0" |
| 13 | Test sequence for the mode Reflectogram |
| 14 | must always be "0" |
| 15 | Enabling noise cancellation |
| 16 | Control of noise cancellation thresholds |
| 17 | Control of noise cancellation thresholds |
| 18 | Control of noise cancellation thresholds |
| 19 | Control of noise cancellation thresholds |
| 20 | Filters - integral phase |
| 21 | Reduce the accuracy of calculations by 10 times |
| 22 | Enable averaging over 8 points along the length |
| 23 | Send all data for the detection algorithm |
| 24 | Disable cyclic algorithm |